VLSI System Design (Graduate Level)

Fall 2020

HOMEWORK I

REPORT

Must do self-checking before submission:

Compress all files described in the problem into one tar

All SystemVerilog files can be compiled under SoC Lab environment

All port declarations comply with I/O port specifications

Organize files according to File Hierarchy Requirement

No any waveform files in deliverables

Student name: 杜冠勳

Student ID: N26094883

| SUMMARY |  |  |
| --- | --- | --- |
| - Requirements | implement a simplified pipeline CPU with the following features - Implement the 33 instructions - The number of pipeline stage is 5. - Register File size: 32x32-bit ➔ x0 is read only 0. - Instruction memory size: 16Kx32-bit  Data memory size: 16Kx32-bit - Timescale: 1ns/10ps - Maximum Clock period: 20ns (50MHz) |  |
| Verification | PROG 0 | Verification for 33 instruction |
|  | PROG 1 | Implentment Bubblesort algorithm for sorting |
|  | PROG 2 | Implentment Multiplication |
|  | PROG 3 | Implentment Greatest common divisor |

| INTRODUCTION FOR DESIGN OF THE CPU |  |
| --- | --- |
| SPEC |  |
| PIPELINE STAGE | 5 |
| INSTRUCTION | 33 |
| FULL GRAPH |  |

| STAGE1 INSTRUCTION FETCH |  |  |
| --- | --- | --- |
| MODULE | FUNCTION | SPECIAL_DESIGN |
| DIDER4 | Get the Instruction Address by dividing the pc_counter result by4 | Using shift right 2 bits instead using the divider which can reduce hardware area. |
| PAUSE INSTRUCTION | If there is any load use data hazard happen,this module will enable to pause the instruction. The instruction an d pc value will be rewrited tio the register in next clock. | NONE |
| rst_contoller | If there is any branch happen this module will enable flush the register. The module will be more detailed discuss later. | NONE |

| STAGE2 INSTRUCTION DECODE & STAGE5 WRITE BACK |  |  |
| --- | --- | --- |
| MODULE | FUNCTION | SPECIAL_DESIGN |
| DECODER | Decoding 33 bits instruction into different elements which is used for the following module | NONE |
| SIGNED UNSIGNED EXTENDED_ SELECT_IMM | Make the Immediate data referring to the type of the instruction | NONE |
| REG | 32×32-bit. registers | Posedge clk write back Negedge clk read data |
| CONTROL | Make the Control signal | NONE |
| rst_contoller | If there is any branch or load use data hazard happen this module will enable flush the register. The module will be more detailed discuss later. | NONE |

| STAGE3 ALU EXECUTION |  |  |
| --- | --- | --- |
| MODULE | FUNCTION | SPECIAL_DESIGN |
| ALU_IN | Data hazard signal will be the input module will decide the input of the alu. | NONE |
| ALU_RD | calculate for the data store in $rd | NONE |
| ALU_ADDR | calculate for the Data memory’s Address which will be stored data next clock. | NONE |
| PC_ADDR | calculate the jumping address and make the signal to enable the system jump or not. | NONE |
| rst_contoller | If there is any branch happen this module will enable flush the register. The module will be more detailed discuss later. | NONE |

| STAGE4 WRITE OR READ MEMORY |  |  |
| --- | --- | --- |
| MODULE | FUNCTION | SPECIAL_DESIGN |
| DIVIDER4 | Divide the address value by 4 Quotient will be the real address value of data memory Reminder will be used for LB SB instruction to load and store data to the corresponding byte. | Using shift right 2 bits instead using the divider which can reduce hardware area. |
| LOW_BYTE_CONTROL | Control for LB SB instruction to load and store data to the corresponding byte. | NONE |
| WB_CONTROL | Wb_control signal will decide data from memrory and alu writing back to rd rigister | NONE |

| OTHER CONTROL MODULE |  |
| --- | --- |
| MODULE | LOAD_DATA_HAZARD |
|  |  | FUNCTION | SPECIAL_DESIGN |
| CONTROL FLOW |  |
| Detect if there is any load use data hazard pc_stall_stage1= (id_exe_read_mem==1'b1 && (if_id_rs1_addr==id_exe_rd_addr ¦¦ if_id_rs2_addr==id_exe_rd_addr))?1'b1:1'b0; Detect if there is any jump signal happened at the same time and prevent pc_stall signal flush the jump action. Jump signal has the priority pc_st all=pc_jump_confirm?1'b0:pc_stall_stage1; instruction_stall=pc_stall; |  |
| MODULE | FORWARDING UNIT |
| CONTROL FLOW |  |
| Detect if there is any data hazard between memory and register $rs1 rs1_mem_hazard=(mem_wb_write_reg==1'b1 && mem_wb_rd_addr!=5'd0 && mem_wb_rd_addr==rs1_addr)?1'b1:1'b0; Detect if there is any data hazard between ALU and register $rs1 rs1_exe_hazard=(exe_mem_write_reg==1'b1 && exe_mem_rd_addr!=5'd0 && exe_mem_rd_addr==rs1_addr)?1'b1:1'b0; Detect if there is any data hazard between memory and register $rs2 rs2_mem_hazard=(mem_wb_write_reg==1'b1 && mem_wb_rd_addr!=5'd0 && mem_wb_rd_addr==rs2_addr)?1'b1:1'b0; Detect if there is any data hazard between ALU and register $rs2 rs2_exe_hazard=(exe_mem_write_reg==1'b1 && exe_mem_rd_addr!=5'd0 && exe_mem_rd_addr==rs2_addr)?1'b1:1'b0; |  |

| MODULE | rst_controller left side (for exe_mem stage and if _id stage) right side (id_exe stage) |
| --- | --- |
| CONTROL FLOW |  |
| (for exe_mem stage and if _id stage) Global rst is cpu’s rst ; jump and branch instruction will make [enable jump signal] active high; if brach conditiction is established then pc_jump control will active high enable to jump address. rst_data=global_rst?1'b1:( enable_jump ? ( pc_jump_control ? local_rst:1'b0) :1'b0); (for id_exe stage) 1.Global rst is cpu’s rst ; 2.Modele will check the pc_stall signal to decide whether pause the pipeline register or not. Next, jump and branch instruction will make [enable jump signal] active high; if branch condition is established then pc_jump control will active high enable to jump address. rst_data=global_rst?1'b1: ( pc_stall ? 1'b1: ( enable_jump ? ( pc_jump_control ? local_rst:1'b0) :1'b0) ); |  |

| MODULE | PC_CONTROL |
| --- | --- |
| CONTROL FLOW |  |
| pc_stall signal will enable rewrite the instruction address again enable_jump signal will enable to jump address. if(pc_stall==1'b1) pc_data=pc; else pc_data=enable_jump?(pc_jump_control?pc_jump_address:next_pc):next_pc; |  |

| R-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION ADD |  | Machine code | Assembly code |
|  |  | 0x006282b3 | add t0 t0 t1 |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | control module(cl ) produce alu_rd_signal(function of the alu);active write_reg signal; and register(rigt)read the data by the address decode by decoder Decoder decode opcode:33 |  |  |
| EXE stage | Alu_in_selector detect the hazard signal produced by forwarding unit and choose the data in memory or alu Alu_rd (ard)calculate the value that will store to the $rd address |  |  |
| MEM stage | By pass the value calculate by Alu_rd Wb_control control the value calculate by Alu_rd to be the write back data. |  |  |
| WB stage | Write_reg signal enable to write back data in register |  |  |

| R-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION AND |  | Machine code | Assembly code |
|  |  | 0x0062f2b3 | and t0 t0 t1 |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | control module(cl ) produce alu_rd_signal(function of the alu);active write_reg signal; and register(rigt)read the data by the address decode by decoder Decoder decode opcode:33,funt3:7,fun7:0 |  |  |
| EXE stage | Alu_in_selector detect the hazard signal produced by forwarding unit and choose the data in memory or alu Alu_rd_control signal=9 execute and function Alu_rd (ard)calculate the value that will store to the $rd address |  |  |
| MEM stage | By pass the value calculate by Alu_rd Wb_control control the value calculate by Alu_rd to be the write back data. |  |  |
| WB stage | Write_reg signal enable to write back data in register |  |  |

| I-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION LW |  | Machine code | Assembly code |
|  |  | 0x0002a283 | Lw t0 0(t0) |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | Decoder decode opcode:3,funt3:2 Control module’output extended_control control the imm data Control module produce read mem signal |  |  |
| EXE stage | Alu_addr calculate the address(alu_addr_out) that is going to load data. Because thers is a load data hazard ,the pc_stall signal active high to pause instruction. |  |  |
| MEM stage | The address is divided by 4 to be the real data memory address The OE signal is active high because it connects with read mem signal Wb_control control the value loaded from memory to be the write back data. |  |  |
| WB stage | Write_reg signal enable to write back data in register The valus is stored in register in 5 stage |  |  |

| I-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION ADDI |  | Machine code | Assembly code |
|  |  | 0xfff00293 | addi t0 t0 -1 |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | Decoder decode opcode:19funt3:0,alu_control:10 Control module’output extended_control control the imm data |  |  |
| EXE stage | Alu_addr calculate the address(alu_addr_out) that is going to load data. The calculation way is selected by alu_control |  |  |
| MEM stage | By pass the value calculate by Alu_rd Wb_control control the value calculate by Alu_rd to be the write back data. |  |  |
| WB stage | Write_reg signal enable to write back data in register The valus is stored in register in 5 stage |  |  |

| S-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION SW |  | Machine code | Assembly code |
|  |  | 0xfe542e23 | Sw t0,-4(s0) |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | Decoder decode opcode:35funt3:2 Control module’output extended_control control the imm data Control module activate the write mem signal |  |  |
| EXE stage | Alu_rd (ard)calculate the value that will store to the $rd address The calculation way is selected by alu_control |  |  |
| MEM stage | The address is divided by 4 to be the real data memory address The 4 bits signal is active low because it connects with read mem signal The data from register is stored in negedge cycle |  |  |
| WB stage | NO ACTION |  |  |

| S-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION SB |  | Machine code | Assembly code |
|  |  | 0xfe542e23 | Sw t0,-4(s0) |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | Decoder decode opcode:35funt3:0 Control module’output extended_control control the imm data Control module activate the write mem signal Control module activate the low byte write signal |  |  |
| EXE stage | Alu_rd (ard)calculate the value that will store to the $rd address The calculation way is selected by alu_control |  |  |
| MEM stage | The address is divided by 4 to be the real data memory address and the written bytes will selected by reminder. The 4 bits signal is 1110 because the low byte control signal only allow write in the low 8bits data. The data from register is stored in negedge cycle |  |  |
| WB stage | NO ACTION |  |  |

| B-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION BEQ |  | Machine code | Assembly code |
|  |  | 0x02628863 | Beq t0,t1 858 |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | Decoder decode opcode:99funt3:0 Control module’output extended_control control the imm data Control module activate the enable jump signal Control module control the alu_pc function with alu pc control signal=1 |  |  |
| EXE stage | Alu_PC determine whether to jump or not. src1!=src2 in this section which will not activate jump signal The next pc address isn’t jump |  |  |
| MEM stage | NO ACTION |  |  |
| WB stage | NO ACTION |  |  |

| B-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION BNE |  | Machine code | Assembly code |
|  |  | 0x 00531c63 | Bne t1,t0,8b8 |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | Decoder decode opcode:99funt3:1 Control module’output extended_control control the imm data Control module activate the enable jump signal Control module control the alu_pc function with alu pc control signal=2 |  |  |
| EXE stage | Alu_PC determine whether to jump or not. src1==src2 in this section which will not activate jump signal The next pc address isn’t jump |  |  |
| MEM stage | NO ACTION |  |  |
| WB stage | NO ACTION |  |  |

| U-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION AUIPC |  | Machine code | Assembly code |
|  |  | 0x 00000297 | Auipc t0 0x0 |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | Decoder decode opcode:23 Control module’output extended_control control the imm data Control module enable the write reg signal Control module control the alu_ed function with alu rd control signal=20 |  |  |
| EXE stage | Alu_in_selector detect the hazard signal produced by forwarding unit and choose the data in memory or alu Alu_rd_control signal=9 execute and function Alu_rd (ard)calculate the value that will store to the $rd address |  |  |
| MEM stage | By pass the value calculate by Alu_rd Wb_control control the value calculate by Alu_rd to be the write back data. |  |  |
| WB stage | Write_reg signal enable to write back data in register The valus is stored in register in 5 stage |  |  |

| U-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION LUI |  | Machine code | Assembly code |
|  |  | 0x 135793b7 | Lui t2 0x13579 |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | Decoder decode opcode:55 Control module’output extended_control control the imm data Control module activate the write reg signal Control module control the alu_rd function with alu rd control signal=21 |  |  |
| EXE stage | Alu_in_selector detect the hazard signal produced by forwarding unit and choose the data in memory or alu Alu_rd_control signal 21 execute and function Alu_rd (ard)calculate the value that will store to the $rd address |  |  |
| MEM stage | By pass the value calculate by Alu_rd Wb_control control the value calculate by Alu_rd to be the write back data. |  |  |
| WB stage | Write_reg signal enable to write back data in register The valus is stored in register in 5 stage |  |  |

| J-TYPE INSTRUCTION VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTION JAL |  | Machine code | Assembly code |
|  |  | 0x0080036f | Jal t1,b10 |
| wave |  |  |  |
| Explanation |  |  |  |
| ID stage | Decoder decode opcode:111funt3 Control module’output extended_control control the imm data Control module activate the write reg signal Control module control the alu_rd function with alu rd control signal=14 Control module control the alu_rd function with alu pc control signal=7 Control module activate the enable jump signal |  |  |
| EXE stage | Alu_in_selector detect the hazard signal produced by forwarding unit and choose the data in memory or alu Alu_rd_control signal=14 execute and function Alu_rd (ard)calculate the value that will store to the $rd address |  |  |
| MEM stage | Alu_PC determine to jump to the address pc+immin the next cylce By pass the value calculate by Alu_rd Wb_control control the value calculate by Alu_rd to be the write back data. |  |  |
| WB stage | Write_reg signal enable to write back data in register The valus is stored in register in 5 stage |  |  |

| SUPER LINT ANALYZE |  |  |
| --- | --- | --- |
| TOTAL LINES:1943 |  |  |
| WARNING:62 |  | 1-62/1943*100%=97% |
| MOST FREQUENT WARNING/ERRORS IN YOUR CODE |  |  |
| Module name not comparing |  |  |
| Modify | For example change cpu_rtl.sv to cpu_rtl |  |
| Default statement isn’t require |  |  |
| Modify | If the case statement is full case delete the default case |  |
| After fixing the problem |  |  |
| WARNING58 |  | 1-58/1943*100%=98% |

LESSON I LEARNED

經由這次的作業 除了複習以前學過的計算機組織架構,也體認到實作時 考慮控制的因素會非常多,例如當我 compile 已經過program0 卻無法過program1 的原因在於當我執行jump指令時,若前一刻有load data hazard 的出現則可能會覆蓋掉 jump訊號 經由修改過後 就可以compile過了,這時我之前學習計組時 沒有想到的問題,而恰巧program0 沒有測試到 LW 接續 JUMP 指令的pattern 導致會有 program 0 過 program 1 沒過的情形發生。此次實作 讓我對CPU架構更為熟悉 獲益良多